# High Level Design Report

# Broadworks Example

Introduction

This report is generated by UCentric to provide details of the existing communications infrastructure, either on-premises (PBX) or cloud-based.

As well as providing details of the existing configuration, this report will provide insights into the considerations you may need to take in account when migrating your existing solution to an alternative provider.

Whilst this report is built from your existing configuration, any such migration plan should be augmented with additional data to provide a detailed plan prior to migration. This would normally include Active Directory, Call Logging (CDR) data, Numbering plans and exports from ancillary systems such as Voice mail, IVR, Call Recorders etc.

# System details

The following audit report has been automatically generated by UCentric Voice Audit from a data capture from the following communications system.

| Name | Broadworks Example |
| --- | --- |
| Type | Broadsoft/Broadworks |
| Audit date | 2024-08 |

# Insights summary

There is a total of 34 Directory Numbers in the existing system, including hunt groups, pickup groups and all physical extensions and profiles. In terms of numbers relating to users, they are as follows:

|  | 0 Profiles |  |
| --- | --- | --- |
|  | 0 Soft phones / Apps |  |
|  | 28 IP Extensions |  |
|  | 0 Digital Extensions |  |
|  | 0 Analog Extensions |  |

Profiles, soft phones and IP extensions are much easier to migrate than older technologies such as digital and analog phones.

# Detailed Insights

This table provides a list of each type of facility that is in use across the existing solution.

Note: Not all items detailed are applicable to all vendors.

| Category | Count | Information |
| --- | --- | --- |
| Duplicate Extns | 0 | Usually caused by both extn and profile having same DN |
| Duplicate names | 0 | Will occur frequently in large organisations |
| Duplicate MAC addresses | 0 | Usually caused by legacy programming |
| Pickup groups with 1 member | 0 | Redundant programming of Pickups |
| Total devices | 23 | Total physical devices |
| Total profiles | 0 | Total profiles |
| Total Home Cluster profiles | 0 | Total profiles marked as `Home Cluster` |
| Total Non-Home Cluster profiles | 0 | Total profiles that are not marked as `Home Cluster` |
| CSF devices | 0 | Total number of devices using `Cisco Client Services Framework` |
| IP Communicators | 0 | Total number of `IP Communicators` |
| Total System Devices | 5 | Total system devices |
| Total Mobility | 0 | Total Mobility Users |
| Total DNs | 34 | Total DNs defined (Extns, Trunks, Groups, Digit Trans, Profiles etc.) |
| Total DNs without name | 5 | Total Directory Numbers without a name defined |
| Total without number | 0 | Total devices or profiles without a valid DN |
| Total Hunt Groups | 1 | Total Hunt Groups in use |
| Total Queues | 0 | Total Queues in use |
| Total ACD Groups | 3 | Total ACD Groups in use |
| Total Workgroups | 0 | Total Workgroups in use |
| Total Paging Groups | 0 | Total Paging Groups in use |
| Total Pickup Groups | 2 | Total Pickup Groups in use |
| Total Soft Phones | 0 | Total Soft Phones in use |

## Migration considerations to Cisco

This table will detail any common facilities being used by a legacy platform and if it is supported in the Cisco hosted and on-premise environments.

To identify parity gaps, the audit is mapped and compared to the Webex platform. There is a total of 34 directory numbers within the audited platform. Some features have a clear parity, and others will need to be reviewed prior to migration.

| Facility in use | Active | Hosted HCS / On-premise CUCM | Webex Calling/ Broadworks |
| --- | --- | --- | --- |
| IP Phones | 27 | Check models | Check models |
| SIP Phones | 16 | Std feature | Std feature |
| ACD Groups | 3 | Std feature | Std feature |
| Pickup Groups | 2 | Std feature | Std feature |
| Cisco Phones | 1 | Std feature | Std feature |
| Hunt Groups | 1 | Std feature | Std feature |
| = Supported = Unsupported = Limited support |  |  |  |

| Overall Feature Parity | Call usage (if available) |
| --- | --- |

## Migration considerations to Microsoft Teams

This table will detail any common facilities being used by a legacy platform and if it is supported in the Microsoft hosted environment.

To identify parity gaps, the audit is mapped and compared to the Teams platform. There is a total of 34 directory numbers within the audited platform. Some features have a clear parity, and others will need to be reviewed prior to migration.

| Facility in use | Active | Teams |
| --- | --- | --- |
| IP Phones | 27 | Check models |
| SIP Phones | 16 | Std feature |
| ACD Groups | 3 | Std feature |
| Pickup Groups | 2 | Std feature |
| Cisco Phones | 1 | Unsupported |
| Hunt Groups | 1 | Std feature |
| = Supported = Unsupported = Limited support |  |  |

| Facility | # in Use | Comment |
| --- | --- | --- |
| No insights found |  |  |

| Overall Feature Parity | Call usage (if available) |
| --- | --- |

| Estimated Monthly Cost of E3 licenses* | Estimated Monthly Cost of E5 licenses* |
| --- | --- |
| £0.00 | £0.00 |

*Based on costs of E3 = £28.10, E3+Voice = £34.10 and E5 = £48.10

## Migration considerations to RingCentral

This table will detail any common facilities being used by a legacy platform and if it is supported in the RingCentral hosted environment.

To identify parity gaps, the audit is mapped and compared to the RingCentral platform. There is a total of 34 directory numbers within the audited platform. Some features have a clear parity, and others will need to be reviewed prior to migration.

| Facility in use | Active | Teams |
| --- | --- | --- |
| IP Phones | 27 | Check models |
| SIP Phones | 16 | Std feature |
| ACD Groups | 3 | Std feature |
| Pickup Groups | 2 | Std feature |
| Cisco Phones | 1 | Check Models |
| Hunt Groups | 1 | Std feature |
| = Supported = Unsupported = Limited support |  |  |

| Overall Feature Parity | Call usage (if available) |
| --- | --- |

# Energy insights

This report details potential carbon impact of the existing solution based on the following assumption: ALL devices are in use for 30% of a 5-day working week

This report is for illustration purposes only.

The current Total kWh of 0.10 can be reflected as 882.15 kW annually – This is equivalent to the following environmental impact.

| 462.77 Kilograms of CO2 | 0.88 Fully grown trees felled | 0.13 Passenger cars driven for 12 months | Annual energy usage of 0.07 of a family homes |
| --- | --- | --- | --- |

Moving to a hosted solution could potentially reduce the carbon impact of your solution by taking advantage of a multi-tenanted platform in an efficient data center.

In the meantime, you can mitigate your impact by…

|  | Recycle 27.35 bags of waste instead of sending them to landfill |
| --- | --- |
|  | Plant 10.32 trees and let them grow for 10 years |
|  | Swapping 23.82 incandescent bulbs to LED equivalent |

But you’d need to do that Every Year

# Handset and Extension Summary

This details a summary of the extension types captured within the audit of the existing voice solution, and the capacity of the existing system if available.

There are 1 extension cards within the system, providing the following capacities:

|  | Capacity | Used |
| --- | --- | --- |
| Analog | 0 | 0 |
| Digital | 0 | 0 |
| Hybrid (i.e. COV) | 0 | 0 |
| IP | 120 | 28 |

There were 0 extensions marked as ‘out of service’

There are 0 analog configured. Any migration to hosted voice that requires analog extensions (e.g. for Fax / Modems / PDQ etc.) will require FXO/Analog Gateways at the required locations.

## Actual utilisation

This table shows all circuit types in use and their actual utilisation after Out-of-Service, and un-numbered extensions and trunks are removed from the programmed counts.

| Circuit type | Capacity | Programmed | Out of Service | No Circuit ID | % Utilised | % Actual utilised |
| --- | --- | --- | --- | --- | --- | --- |
| Extns - IP | 120 | 29 24.17% | 0 0.00 % | 1 1% | 24.17% | 23.33% |

## Extension Number (DN) Ranges

This details all of the extension number ranges that have been determined within the existing solution, and the number of extensions or users within each range. This is useful to determine the need for DID numbering, or to properly plan SBCs or Voice Gateway routes.

| Prefix | Length | Total allocated | Start number | End number |
| --- | --- | --- | --- | --- |
| 02** | 4 | 27 | 0360 | 0391 |
| 54** | 4 | 1 | 4321 | 4321 |

If you have extensions with different digit lengths, this can cause issues on a hosted voice solution, and should be reviewed prior to migration.

## Handset Types

Where known, this details all the different handsets or softphone types in use. This allows you to understand what features and facilities users are currently utilising and ensuring that any migration does not reduce their experience.

| Handset | Model | Programmed | OOS | No Extn # | Details |
| --- | --- | --- | --- | --- | --- |
|  | Generic SIP Phone | 11 | 0 | 0 | IP Handset |
|  | Generic SIP Phone-Connected identity | 5 | 0 | 0 | IP Handset |
|  | Snom-D713 | 3 | 0 | 0 | Not specified |
|  | Snom-D785 | 1 | 0 | 0 | Not specified |
|  | Snom-M400 | 1 | 0 | 0 | Not specified |
|  | Business Communicator - PC | 1 | 0 | 0 | Not specified |
|  | Cisco 7960 | 1 | 0 | 0 | IP Handset. LCD. 6 soft keys. Full duplex speakerphone. Headset. |

# Call Flows, Routing and Groups

## Hunt, Pickup and ACD Groups

Hunt groups and pickup groups operate in two different ways; Hunt and ACD groups will find the first available member (based on rules applied), whereas Pickup groups ring all members at the same time.

| Category | Pilot Number | Name | # of Members | Warning |
| --- | --- | --- | --- | --- |
| ACD | 0277 | MITCallCenter | 1 | Single member of group |
| ACD | 5421 | MITabc | 1 | Single member of group |
| ACD | 0270 | CallCenter-Premium | 1 | Single member of group |
| Hunt | 0275 | MIT_HG | 3 |  |
| Pickup | CallPickup | CallPickup | 8 |  |
| Pickup | MIT_TEST | MIT_TEST | 2 |  |

## Groups with a single member

| Category | Name | Type | Member DN | Member Name |
| --- | --- | --- | --- | --- |
| 0270 | CallCenter-Premium | ACD | 0266 | Grace West |
| 0277 | MITCallCenter | ACD | 0268 | Marty McFly |
| 5421 | MITabc | ACD | 0284 | Danny Harper |

- Note: Pickup groups with a single member are redundant – Hunt groups with a single member may be used for diverting non-geographic or other external numbers and should be reviewed.

# Region information

Many systems provide a centralised call-control, with gateways to extend that service to remote offices. Detailed below is a summary of any regional or remote users discovered.

| Region ID | Region Name | Standard Sets | IP Sets |
| --- | --- | --- | --- |
| 1 | MIT-Grp1 | 0 | 4 |
| 2 | MIT-Grp2 | 0 | 1 |

## Extension ranges by location

Much like the previous list of extension ranges, this shows which extension numbers are being used, but in this case, broken down by region or location.

| Region | Extn prefix | Total # in range | Range start | Range end |
| --- | --- | --- | --- | --- |
|  | 02** | 23 | 0261 | 0291 |
| MIT-Grp1 | 02** | 4 | 0260 | 0277 |
| MIT-Grp2 | 54** | 1 | 5421 | 5421 |

# Glossary of terms

| Term | Definition |
| --- | --- |
| ACD group | Automatic Call Distribution – a type of hunt group that can route calls based on rules (e.g. Longest ringing, Skills, calling-line ID etc.) |
| Analog Adapter | An Analog Telephone Adapter device (e.g. connector to Fax, Intercom, Alarm, etc.) – Sometimes called an FXO/FSX adapter |
| Analog Phone | An Analog Telephone device that is connected to a remote gateway (e.g. connector to Fax, Intercom, PDQ, Alarm, etc.). |
| Audio Conference Phone | An Audio Conference desk phone device. |
| Common Area Phone (CAP) | A device that is not associated with a user. Also referred to as Standalone Device or Utility Phone |
| CTI | Computer Telephony Integration (CTI) enables users to take advantage of computer-processing functions while making, receiving, and managing telephone calls. CTI applications allow you to perform such tasks as retrieving customer information from a database using a caller ID, or to work with the information gathered by an Interactive Voice Response (IVR) system to route a customer’s call, along with their information, to the appropriate customer service representative. |
| Device Profile | A device profile comprises the set of attributes (services and/or features), e.g. line number, forwarding, that will appear on the phone when the associated user logs in. |
| Devices | Hardware or software telephony clients configured in the legacy PBX. Devices can be associated to one or multiple extensions. |
| Feature Parity | This shows if particular features can translate easily to an alternative provider |
| FXO / FXS | Foreign Exchange Subscriber/Office – This is an analog line or port that a phone or fax machine connects to. It is a common description for Analog ports on a Cisco CUCM/CME |
| Hunt Group | A Hunt Group is the method of distributing phone calls from a single telephone number to a group of several phone extensions. |
| IP Phone | A desktop phone also referred to as a handset device. |
| Legacy PABX/PBC | The telephony system which is being assessed and reported in this document. |
| Line | A telephone number configured on the legacy PBX. |
| Orphaned Cisco End-User | An end-user that is not associated with any device or device profile. |
| On-Prem | Another definition for locally hosted voice (as opposed to ‘Cloud’ hosted) |
| Partial Feature Parity | This indicates that the feature is currently not fully supported by Microsoft Teams. Users configured with this feature can be migrated but might lose some functionality. In some cases, 3rd party solutions can close the gap. |
| Pickup Group | Group of users authorized to answer calls to a telephone extension within that group of users. |
| Software Client | An Android / iOS / PC Software client device. |
| Standalone Device | A device that is not associated with a user. Also referred to as Common Area Phone in Microsoft Teams. |
| Telepresence | An Advanced Video Conferencing device. |
| Users | End users that are configured on the legacy PBX. Users can be associated to multiple devices and device profiles. |
| Video Phone | A handset device with video capabilities. |
| Wireless IP Phone | A wireless handset device. |